Artesian Streams · Ring PoC

Hands-on Build Instructions

Parts 1, 2, 3, 4, 6 and 10, broken down step by step

This document is the expanded version of docs/08-build-guide.md for the parts you are doing yourself:

- Part 1 · What you have and what to buy

- Part 2 · Set up your computer

- Part 3 · Prepare the sensors (reflash and solder the reset pin)

- Part 4 · Stage 1 wiring (sensors to the ESP32) and first power-up

- Part 6 · Mount the sensors on the rig

- Part 10 · Stage 2 wiring (the LED ring)

While you work through these, I build the live site (Ring Studio), then the Phase 0 firmware (Part 5) and the Phase 1 firmware (Part 7). Each time I push an update I will tell you, and the loading steps are short: git pull, then the two PlatformIO upload buttons.

Version: 30 September 2026. The repo copy of this file is docs/08a-hands-on-instructions.docx. If the repo and this file ever disagree, the repo's markdown docs win, and tell me so I can fix this file.

Contents

If the contents list shows as empty, right-click it in Word and choose Update Field.

# How to use this document

Do the steps in order within each part. Every step has numbered actions. Do one action, look for the result it describes, then do the next one. Do not skip ahead when something does not match: stop, and use the fix code the step points to (Appendix A has the codes you are most likely to need; the full set is docs/09-troubleshooting.md).

### Three rules that prevent almost every fault

- Everything unplugged while you wire. No USB cable in the ESP32, LED supply off, before you touch a wire.

- Count pins from the top of each header, with the board the same way up as the diagram. Never count with the board flipped over.

- A sensor's own USB port and the ESP32 wires are never connected at the same time. Reflash a sensor only with all its wires off.

### Checkpoints

A checkpoint is where you send me photos or results and wait. They exist so I can catch a mistake before it costs an hour, or a board. There are five in these parts: after Step 3, Step 11, Step 14, Step 19 and Step 41.

### The order across parts

| Part | When | Depends on |
| --- | --- | --- |
| 1 | Today | nothing |
| 2 | Today or tomorrow | nothing |
| 3 | As soon as CubeProgrammer and the firmware file are on the PC | Part 2 |
| 4 | Right after Part 3 | Parts 2 and 3 |
| 5 (mine, then yours) | After Checkpoint 4. I push the Phase 0 firmware; you load it and run bench tests T1 to T4 | Part 4 |
| 6 | After Part 5's bench tests pass, so wiring faults are found on the desk, not inside the rig. The backboard, foil shields and mount printing can be done any time | Part 5, plus the measurements in Step 24 |
| 10 | When the ordered parts arrive. Any time after Part 4. The LED tests (C0) arrive with the Phase 1 firmware | Part 4, plus the Step 4 order |

### Tools you need on the bench

☐  Soldering iron (temperature-controlled if you have one, set 320 to 350 °C), solder (0.8 mm or thinner, leaded or lead-free), a damp sponge or brass tip cleaner

☐  Small side cutters and wire strippers (for 18 AWG and for thin hookup wire)

☐  Permanent marker, tape, a tape measure with millimetres, a ruler

☐  Phone camera for checkpoint photos

☐  A 5 mm drill or a leather punch for the calibration template holes (Part 8, later)

☐  Hot glue gun or double-sided foam tape (Part 6 mounting)

☐  Heat-shrink tubing or electrical tape (Part 10 joints)

# Part 1 · What you have and what to buy

You do this   ·   about 30 minutes at the desk

## Step 1 · Lay out what you already have

Put everything on the bench and tick each line. The descriptions help you confirm you have the right item.

☐  ESP32-S3 board. Black, about 70 × 28 mm, a silver metal can at one end printed ESP32-S3 and two USB-C ports at the other end. On the underside the ports are marked COM (or UART) and USB. Two tiny buttons near the ports, marked BOOT (or B) and RST (or R).

☐  Sensor board 1. Red SparkFun board, about 25 × 40 mm, a USB-C port at one end, a small dark blue/black module (the radar) on the front, two rows of holes down the sides, two tiny buttons at the end opposite the USB.

☐  Sensor board 2. Same as above.

☐  5 V LED strip coil (the one whose label or box says 5V, WS2812B, or "addressable").

☐  12 V LED strip coil (kept aside; we will almost certainly use the 5 V one).

☐  LED power supply (brick or open-frame supply, with a sticker).

☐  A USB-C cable that carries data. A cable from a phone or laptop box usually does. Cheap short cables sold for charging often do not; you find out in Step 9.

☐  Female-to-female jumper wires, at least 10, ideally in several colours.

☐  Soldering iron and solder, and two spare header pins (a strip of 0.1 in / 2.54 mm male pins you can snap two off).

## Step 2 · Read the LED strip labels

- Unroll about 30 cm of the 5 V coil. Lay it flat with the LEDs facing you.

- Find a cut point: a set of copper pads across the strip with a small scissors symbol or a dashed line, every 1 to 3 LEDs.

- Read the tiny letters printed beside the pads. Write them down exactly. What they mean:

  - 5V, +5V, VCC, VDD or +: power.

  - DIN, DI, DAT, D or DO: data. DIN/DI is the input side of a pad set, DO the output side.

  - GND, G or -: ground.

  - BI or BIN as a fourth pad: a backup data line (WS2813 type). Still fine; tell me.

  - R, G, B and +12V/+5V as four pads: a plain RGB strip, not addressable. Not usable here. Tell me.

- Look for tiny arrows printed along the strip between LEDs. They show the data direction. Note which way they point relative to the coil's loose end.

- Count the LEDs in 10 cm and multiply by 10. That is the LEDs per metre (30, 60 or 144). Write it down.

- Photograph the pads close up with the letters readable, and one photo showing the arrows.

- Do the same for the 12 V coil (labels, arrows, LEDs per metre, photos).

## Step 3 · Read the power supply label

- Find the sticker on the LED power supply, usually the bottom or the back.

- Find the line starting OUTPUT. It reads like OUTPUT: 5V ⎓ 4A or DC 5V 5A. We need 5 V and 4 A or more. Write down both numbers.

- Look at how the output comes out of the supply. It is one of these; note which:

  - Two bare wires (red and black, or with a white stripe for positive): easiest.

  - A round barrel plug: you need a "female DC barrel jack to screw terminal adapter" (usually 5.5 × 2.1 mm). Add it to the order in Step 4.

  - Screw terminals on a metal open-frame supply with terminals marked L, N and ⏚ on one side and +V, −V on the other: the L/N side is mains voltage. Only wire the mains side if you are confident doing so, with the cable clamped and the case earthed; otherwise buy an enclosed 5 V 5 A brick instead.

- Photograph the sticker and the output connector.

CHECKPOINT 1

Stop here. Send me the following and wait for my reply before you go on:

- the 5 V strip pad photo and what the letters say;

- the 12 V strip pad photo;

- the LEDs per metre for each strip;

- the supply sticker photo and the connector type.

I will confirm which strip to use, whether the supply is big enough, and whether anything needs adding to the order.

## Step 4 · Order the parts

Order now; nothing in Parts 2 to 5 needs them, so keep going while they ship. Search terms are given so the listing you find matches.

| # | Search for | Qty | Why | Notes |
| --- | --- | --- | --- | --- |
| 1 | SparkFun Qwiic cable female jumper 4-pin | 3 | Neat sensor cables for mounting (Part 6) | Any 4-core cable with female 0.1 in ends works. Under 50 cm |
| 2 | 74AHCT125 DIP-14 | 2 | LED data level shifter, 3.3 V to 5 V | One spare. Must be AHCT, not HC or HCT |
| 3 | 330 ohm resistor 1/4 W | 1 pack | LED data line | Colour bands orange-orange-brown |
| 4 | 10k ohm resistor 1/4 W | 1 pack | Pull-down on the level shifter input | Colour bands brown-black-orange |
| 5 | 1000uF 16V electrolytic capacitor | 1 pack | Smooths LED power at the strip input | 10 V rating is also fine |
| 6 | inline blade fuse holder 18 AWG | 1 | Protects the LED power | Comes with wire tails |
| 7 | 5A blade fuse (ATC/ATO or mini, to match the holder) | 2 | Fuse plus a spare |  |
| 8 | 18 AWG red black wire (zip cord) | 3 m | LED power to both strip ends | Stranded, not solid |
| 9 | half size breadboard 400 point | 1 | Holds the level shifter chip | With power rails along the edges |
| 10 | breadboard jumper wire kit male-male | 1 kit | Short wires for the chip tie-offs |  |
| 11 | SparkFun XM125 (SEN-24540 or the current part number) | 2 to 3 | Spare sensors: demo insurance | Each spare is reflashed on arrival (Step 13) |
| 12 | ping pong balls | 1 pack | Calibration wand (Part 8) |  |
| 13 | 1/4 inch wooden dowel 24 inch | 1 | Calibration wand handle |  |
| 14 | poster board 24 x 24 inch (or larger) | 1 | Calibration template (Part 8) | Stiff card; foam board also works |
| 15 | aluminium foil tape | 1 roll | Shields behind the sensors (Part 6) | Kitchen foil and glue also works |
| 16 | 5V 2A USB-C wall adapter | 1 | Powers the ESP32 at demos with no laptop | Only if you do not have one |
| 17 | switched power strip | 1 | One switch for the LED supply and the ESP32 adapter | Only if you do not have one |
| 18 | DC barrel jack female to screw terminal 5.5x2.1 | 1 | Only if your supply has a barrel plug (Step 3) |  |
| 19 | foam board 24 x 36 inch or plywood offcut | 1 | Backboard behind the rig (Part 6) |  |

Note Nothing here is critical to Parts 2 to 5. If a listing is out of stock, buy the nearest equivalent and tell me what you bought. For the 3D-printed sensor mounts (Part 6): if you do not have access to a printer, Step 25 has a no-printer alternative using a small wood block and hot glue. Tell me which you will use and I design for it.

# Part 2 · Set up your computer

You do this   ·   about 45 minutes, most of it waiting for installers

These steps are for Windows 10 or 11. Every command is copied into PowerShell exactly as written. To paste into PowerShell, right-click inside the window.

## Step 5 · Open PowerShell

- Click Start (the Windows logo at the bottom of the screen).

- Type PowerShell.

- Click Windows PowerShell (or Terminal; either is fine). A blue or black window opens with a line ending in >. That is where you paste commands.

## Step 6 · Install Git, VS Code and Python

- Click inside the PowerShell window.

- Copy the line below, right-click in the window to paste it, and press Enter:

winget install --id Git.Git -e

- If it asks you to agree to source terms, type Y and press Enter. A progress bar runs, then it prints Successfully installed. This takes 1 to 3 minutes.

- Repeat with the next line. Wait for Successfully installed each time before pasting the next.

winget install --id Microsoft.VisualStudioCode -e

winget install --id Python.Python.3.12 -e

- If PowerShell says winget is not recognized: open the Microsoft Store, search App Installer, click Update (or Get), wait for it to finish, close PowerShell, open it again (Step 5) and repeat this step from action 2.

- If a line says the program is already installed, that is fine; go to the next line.

## Step 7 · Check the installs

- Close PowerShell (click the X). Open a new one (Step 5). Newly installed programs are only found in a new window.

- Paste each line and press Enter. Each should print a version number, for example git version 2.47.0, Python 3.12.6, and three lines for VS Code.

git --version python --version code --version

- If python --version opens the Microsoft Store instead of printing a version: close the Store, then run winget install --id Python.Python.3.12 -e again, close PowerShell, open a new one and retry.

- If any line prints an error in red, stop and send me a screenshot of the window.

- Install the PlatformIO add-on for VS Code and the maths library used by the simulation tools:

code --install-extension platformio.platformio-ide python -m pip install numpy

- The first line prints Extension 'platformio.platformio-ide' ... was successfully installed. The second ends with Successfully installed numpy.

- Open VS Code once now: type code and press Enter. In the bottom-right a notice says PlatformIO IDE: installing PlatformIO Core. Leave it for 2 to 5 minutes until it says PlatformIO IDE installed successfully, then click Reload Now. If it asks to install Python or Git for PlatformIO, click Install.

- Close VS Code.

## Step 8 · Download the project

- In PowerShell, paste these lines one at a time, pressing Enter after each:

New-Item -ItemType Directory -Force "$HOME\code" ¦ Out-Null cd "$HOME\code" git clone https://github.com/lorops-stack/artesian-ring-poc.git

- After the third line a GitHub sign-in window opens. Click Sign in with your browser, sign in as lorops-stack, and click Authorize. Go back to PowerShell.

- It prints Cloning into 'artesian-ring-poc'... and finishes in a few seconds. You now have the folder C:\Users\<you>\code\artesian-ring-poc. Open it in File Explorer to check it exists and has a docs folder inside.

Getting my updates later Whenever I tell you pushed, get my update with these two lines in PowerShell: cd "$HOME\code\artesian-ring-poc" then git pull If git pull refuses with Your local changes would be overwritten, see fix U7 in Appendix A.

## Step 9 · Check the USB drivers

- Press Windows key + X, then click Device Manager.

- Find Ports (COM & LPT) in the list and click the small arrow to the left of it so it opens. If there is no such entry yet, that is normal; it appears when a device is plugged in. Leave the window open and visible.

- Plug the ESP32 into the PC with your data cable. Use the USB-C port marked COM (or UART) on the underside of the board.

- Within a few seconds a new line appears under Ports, like USB-SERIAL CH343 (COM5). Write down the COM number. You will need it later.

- If it does not look like that:

  - A new line appears but does not say CH343: unplug and try the ESP32's other USB-C port. Use whichever port shows CH343 and note which it is (left or right when the USB ports face you).

  - Nothing appears at all: try a different cable. Then a different USB port on the PC. See fix W8.

  - The line has a yellow warning triangle: open your browser, go to wch-ic.com, search CH343SER, download it, run it and click INSTALL. Then unplug and replug the ESP32. The triangle goes away.

- Unplug the ESP32.

- Plug in one sensor board by its own USB-C port. A line like USB-SERIAL CH340 (COM6) appears. Write down that COM number too. If it has a yellow triangle, download and install CH341SER from wch-ic.com the same way, then unplug and replug.

- Unplug the sensor. Plug in the other sensor and check it also appears (it may take the same or a different COM number). Unplug it.

## Step 10 · Install STM32CubeProgrammer

This is the tool that loads the new program into the sensors (Part 3).

- In your browser, search STM32CubeProgrammer download and open the st.com result (the page is titled STM32CubeProg).

- Scroll to Get Software and click Get latest on the Windows row (STM32CubePrg-W64).

- It asks you to log in or register. Click Login/Register, create a free account (a confirmation email arrives; click its link), then come back and click Get latest again. The download is a zip of about 300 MB.

- When it finishes, right-click the zip in your Downloads folder, choose Extract All, then Extract.

- Open the extracted folder and double-click SetupSTM32CubeProgrammer_win64.exe. Click Next through every page with the default choices, accept the licence, and click Done.

- If Windows asks to install a USB driver during the setup, allow it.

- Check: click Start, type STM32CubeProgrammer, and it appears. Do not open it yet.

## Step 11 · Get the sensor firmware file

- In your browser, search Acconeer developer site and open developer.acconeer.com.

- Click Log in / Sign up and create a free account (confirm the email).

- Go to Downloads (or Software downloads). Find the XM125 section and download the XM125 software package for the A121 sensor (the name looks like acconeer_xm125_a121_software_v1.x.zip or similar).

- Right-click the downloaded zip, Extract All, Extract.

- Open the extracted folder. In the search box at the top right of File Explorer, type i2c_distance_detector and press Enter.

- The file i2c_distance_detector.bin appears (there may also be .hex or .elf versions; you want the .bin). Right-click it, choose Open file location, then click in the address bar at the top, copy the path and paste it somewhere you can find (a note on your desktop).

- If several .bin files with that name are in different folders, use the one in a folder named for the XM125 (not another module).

CHECKPOINT 2

Stop here. Send me the following and wait for my reply before you go on:

- Steps 7 to 11 worked (a one-line message is fine);

- the ESP32 appears as CH343 on a COM port, and which USB-C port did it;

- the COM numbers you wrote down (ESP32 and sensors);

- the folder path of i2c_distance_detector.bin (or a screenshot of the extracted folder if you cannot find it).

# Part 3 · Prepare the sensors

You do this   ·   about 45 minutes

The sensors ship with the presence detector program. This project needs the distance detector, so each board gets reflashed once. Then one header pin is soldered into each board's RST hole so the ESP32 can reset a sensor that hangs.

## Step 12 · Label the sensors

- Take the two sensor boards.

- With a permanent marker, write A on the back of one and B on the back of the other (the back is the side without the blue radar module). Make it large.

- From now on A is always A. A and B end up at different corners with different wiring, so the labels matter.

## Step 13 · Reflash sensor A

Before you start Nothing else may be plugged into the PC's USB during this step, and the sensor must not be wired to the ESP32.

- Plug sensor A into the PC by its own USB-C port. Its small PWR light comes on.

- Put it into loading mode. The two tiny buttons are at the end opposite the USB, marked BOOT and RST:

  - Press and hold BOOT.

  - While holding BOOT, press and release RST.

  - Release BOOT.

  - Nothing visible changes. That is normal.

- Click Start, type STM32CubeProgrammer, press Enter. The program opens with a dark window and a blue panel on the right.

- In the blue connection panel on the right:

  - The drop-down at the top says ST-LINK. Click it and choose UART.

  - Port: click the drop-down and pick the COM number sensor A had in Step 9 (for example COM6). If the list is empty, click the small circular refresh icon beside it, then look again.

  - Baudrate: 115200.

  - Parity: leave as Even. Data bits 8, stop bits 1.0, flow control Off (the defaults).

  - Click the green Connect button.

- Watch the Log at the bottom. Success looks like Serial Port COM6 is successfully opened and Device ID : 0x..., and the Connect button changes to Disconnect.

- If it says Unable to connect or No response:

  - Redo the button sequence (action 2), then click Connect again straight away. The board only waits in loading mode until it is reset.

  - Check the Port is the sensor's COM number and the interface is UART.

  - Still failing: fix F4 in Appendix A.

- On the far left, click the second icon from the top: a green down-arrow into a chip. Its tooltip says Erasing & Programming.

- Next to File path, click Browse. Find and select i2c_distance_detector.bin (Step 11), and click Open.

- Start address: click in the box and type 0x08000000. That is zero, x, zero, eight, then six zeros. Check it character by character.

- Tick Verify programming if it is not ticked. Leave Run after programming unticked.

- Click Start Programming. A progress bar runs for 10 to 30 seconds.

- A box saying File download complete appears. Click OK. The log also says Download verified successfully.

- Click Disconnect (top right).

- Press the sensor's RST button once so it starts the new program.

- Unplug sensor A.

If Start Programming fails with an error: fix F4, second part ("Connects but programming fails").

## Step 14 · Reflash sensor B

- Plug in sensor B on its own.

- Repeat Step 13 exactly, actions 2 to 15. Only the COM number may differ.

When the spare sensors arrive, label them SP1, SP2, SP3 and reflash each one the same way, the day they arrive.

CHECKPOINT 3

Stop here. Send me the following and wait for my reply before you go on:

- that both boards showed File download complete (one line is fine);

- if either failed: a screenshot of the whole CubeProgrammer window, including the log at the bottom.

## Step 15 · Solder one pin into each sensor's RST hole

The ESP32 must be able to hard-reset a sensor that stops answering (bench test T14 checks this). That needs one wire to each sensor's RST hole, so the hole gets a header pin.

- Snap two single pins off the header strip (bend the plastic between two pins; it snaps cleanly).

- Hold sensor A the same way as in the diagrams: USB-C at the top, buttons at the bottom, blue module facing you.

- Look at the left row of holes (the row without pins on your boards). Counting from the top, the holes are: INT, 3V3, 1V8, VU, G, G, SIO, CLK, RST. The bottom hole is RST, and it is labelled RST in tiny print beside it.

- Push a header pin into that hole so the long end points the same way as the pins already soldered on the right side (normally down, out of the back of the board). The short end sits in the hole and pokes just above the top surface.

- Tip for a straight pin: push the pin's long end into a breadboard first, then lower the sensor onto it and let the board rest on the pin's plastic collar. The board holds itself level while you solder.

- Heat the iron to about 330 °C. Wipe the tip, add a touch of solder to it so it is shiny (tinning).

- Touch the tip to both the pin and the copper ring of the hole at the same time for 2 seconds, then feed in a small amount of solder at the joint (not onto the iron). Hold for 1 more second, remove the solder, remove the iron. Do not move the board for 3 seconds.

- Look at the joint. Good: a small shiny cone hugging the pin. Needs redoing: a dull grey ball, or solder sitting on top of the pin without touching the ring (reheat it for 3 seconds with a touch of fresh solder). Also check the solder has not bridged to the hole above (CLK). If it has, reheat and drag the excess away with the tip.

- Let it cool for a minute. Gently wiggle the pin: it must not move.

- Repeat actions 2 to 9 on sensor B.

- Do not add anything to the WU pin on the right side. It stays unconnected.

Care The blue module on the front of each board is the radar. Never touch it with the iron or with foil. Keep solder splashes and flux off it.

# Part 4 · Stage 1 wiring and first power-up

You do this   ·   about 30 minutes

Ten jumper wires connect the two sensors to the ESP32. Each sensor has its own pair of data wires, its own reset wire, and power. This is the diagram to keep beside you (it is also in the repo as docs/wiring-stage1-sensors.png, so you can zoom in on a screen):

Stage 1: the two sensors on the ESP32. Boards drawn component side up, exactly as they sit on the bench.

## Step 16 · Set up the bench

- Unplug everything from the PC. No USB cable in the ESP32 or the sensors.

- Lay the ESP32 on the table component side up: the silver module at the top, the two USB-C ports at the bottom. This is the view in the diagram.

- The pins stick out underneath. You plug the female jumper ends onto them from below without flipping the board over. Lift the board's edge slightly, slide a wire on, lower it. If you find you must turn the board over, put a piece of tape on the underside marked LEFT first, because a flipped board swaps its left and right sides and every pin count would be wrong.

- Lay sensor A above-left of the ESP32 and sensor B below-left, both with their USB-C at the top and their blue module facing up, as in the diagram.

- Sort your jumper wires: 2 red, 2 black, 2 blue, 2 yellow, 2 purple if you have them. If not, any colours; write the wire number on a piece of tape on each.

## Step 17 · Connect the 10 wires, one at a time

For each wire: read its row, count the pin position down from the top of that header, push the wire on firmly at both ends, then tick the row. Do not pick up the next wire until the row is ticked.

| Tick | Wire | ESP32 end | Sensor end | Colour |
| --- | --- | --- | --- | --- |
| ☐ | W1 | Left header, pin 1 (the very top, 3V3) | Sensor A, right side, pin 8 (3V3) | red |
| ☐ | W2 | Left header, pin 2 (3V3) | Sensor B, right side, pin 8 (3V3) | red |
| ☐ | W3 | Left header, pin 22 (the very bottom, GND) | Sensor A, right side, pin 7 (G) | black |
| ☐ | W4 | Right header, pin 1 (the very top, GND) | Sensor B, right side, pin 7 (G) | black |
| ☐ | W5 | Left header, pin 12 (GPIO8) | Sensor A, right side, pin 9 (SDA) | blue |
| ☐ | W6 | Left header, pin 15 (GPIO9) | Sensor A, right side, pin 10 (SCL) | yellow |
| ☐ | W7 | Left header, pin 10 (GPIO17) | Sensor B, right side, pin 9 (SDA) | blue |
| ☐ | W8 | Left header, pin 11 (GPIO18) | Sensor B, right side, pin 10 (SCL) | yellow |
| ☐ | W9 | Left header, pin 6 (GPIO6) | Sensor A, left side, pin 9 (the bottom, RST: the pin you soldered) | purple |
| ☐ | W10 | Left header, pin 7 (GPIO7) | Sensor B, left side, pin 9 (the bottom, RST) | purple |

Counting on the ESP32's left header, from the top: 1 = 3V3, 2 = 3V3, 3 = RST, 4 = GPIO4, 5 = GPIO5, 6 = GPIO6, 7 = GPIO7, 8 = GPIO15, 9 = GPIO16, 10 = GPIO17, 11 = GPIO18, 12 = GPIO8, 13 = GPIO3, 14 = GPIO46, 15 = GPIO9, 16 = GPIO10, 17 = GPIO11, 18 = GPIO12, 19 = GPIO13, 20 = GPIO14, 21 = 5Vin, 22 = GND. The numbers are also printed on the board beside each pin; use them to confirm your count.

Counting on a sensor's right header, from the top (USB end): 1 = BOOT, 2 = IO1, 3 = IO0, 4 = TX, 5 = RX, 6 = ADDR, 7 = G, 8 = 3V3, 9 = SDA, 10 = SCL, 11 = WU. WU (pin 11) gets no wire. The labels are printed on the board next to each pin.

Watch out Pins 13 and 14 on the ESP32's left header (GPIO3 and GPIO46) sit between W5 and W6. They must stay empty. A wire that lands there by miscounting can stop the ESP32 from loading firmware. The two sensors' wires must never be crossed: A's four wires (W1, W3, W5, W6, W9) all go to board A; B's (W2, W4, W7, W8, W10) all go to board B.

## Step 18 · Check every wire

- Go down the table again, wire by wire, in order. Put a finger on the ESP32 end, follow the wire to the sensor end, and read the label printed next to the sensor pin. It must match the table.

- Check specifically for the two most common mistakes: SDA and SCL swapped (W5/W6 or W7/W8), and a wire one pin off on the ESP32's left header.

- Check that no bare metal from two wire ends touches, and that no wire end is only half on its pin.

- Compare your bench with the diagram picture one last time.

## Step 19 · First power-up

- Plug the ESP32's COM port (the one that showed CH343 in Step 9) into the PC.

- Look at each sensor. Its small PWR light (near the USB-C, top left) must be on. If one is off: unplug the ESP32 and go to fix W6.

- After 10 seconds, touch each board lightly with a fingertip. Warm is fine. Hot, or any smell: unplug immediately and go to fix W7. Do not power up again until I have seen photos.

- In Device Manager the CH343 COM port appears as before. If it appears and disappears repeatedly: unplug, fix W8 point 5.

- Find the small RGB status light on the ESP32 near the USB ports (a tiny white square, not the red power LED). With no firmware yet it may stay dark; that is fine for now. But check the tiny pair of solder pads marked RGB beside it:

  - If the two pads are joined by a blob of solder, nothing to do.

  - If they are two separate bare pads with a gap: unplug the board, and bridge them with a small blob of solder (touch the iron across both pads, feed a little solder, remove). Without this bridge the status light never works (fix U1).

- Plug back in if you unplugged, and leave it powered.

CHECKPOINT 4

Stop here. Send me the following and wait for my reply before you go on:

- one clear photo from directly above the bench showing all 10 wires and both boards;

- whether both PWR lights are on;

- whether the RGB pads were already bridged or you bridged them.

After this checkpoint I push the Phase 0 firmware (Part 5). You load it with the steps in 08-build-guide.md Steps 21 to 23 and run bench tests T1 to T4. Then Part 6.

# Part 6 · Mount the sensors on the rig

You do this   ·   about 1 to 2 hours, plus printing time

Do this after Part 5's bench tests pass (Checkpoint 5), so any wiring fault is found on the desk. The backboard, the foil shields and the mount printing can be done earlier.

## Step 24 · Measure the ring for the mount design

I do this   ·   about 15 minutes

I design the mounts around your ring. I need these measurements and photos. Measure in millimetres.

- Sink opening: confirm the inside width and depth at the top (design value 23 × 21 in = 584 × 533 mm). Measure both and write the real numbers.

- Ring cross-section at a back corner: the width of the ring's top surface (from the sink edge outward), and its height above the counter. Photograph the corner from above and from the side with a ruler in the shot.

- Back corners: the shape of the corner (square, rounded and its radius), and whether there is flat space on top of the ring at the corner at least 30 × 45 mm for a mount to sit on.

- Sink depth: from the ring's top surface down to the basin bottom at the centre.

- Behind the ring: how much space there is behind the back edge (to a wall or the counter edge) for the backboard and cables.

- Printer: whether you have a 3D printer or a print service available, and its bed size. If not, say so; I design the no-printer version (a wood block and hot glue, Step 25 option B).

- Send me the numbers and photos. I send back the mount design (an STL file for printing, or a cutting sketch for option B) within the day.

Design intent Each mount holds the board upright (vertical), with the blue module facing into the sink, turned 45° inward toward the sink centre and tilted about 20° downward into the basin. The tilt must be adjustable by a few degrees, because it gets tuned on the bench. The module centre should sit level with the top of the ring, or as close above it as the mount allows, at the back-left and back-right corners. Both sensors at the same height (within 20 mm; fix P9).

## Step 25 · Make and fit the mounts

### Option A · 3D printed

- Print two mounts from the STL I send (PLA or PETG, 0.2 mm layers, 3 walls, 20% infill is plenty). Print a third if you want a spare.

- Clean any stringing from the slot where the board sits. Test-fit a sensor: it should slide in and hold without force. If it is tight, sand the slot lightly; do not force the board.

- Fit the mounts to the ring's back-left and back-right corners with the fixing I specify (double-sided foam tape, screws or hot glue depending on the ring material). Mark their positions with tape first so both are the same distance from the back edge.

### Option B · No printer

- Cut two wood blocks about 40 × 30 × 25 mm (a pine offcut or a paint stirrer stack glued together).

- Cut a saw slot across the top of each block, 1.7 mm wide (one hacksaw cut, opened slightly with a second pass) and 6 mm deep, so the sensor board stands upright in it.

- Angle the slot 45° to the block's front edge so that, with the block square to the ring corner, the board faces the sink centre. Check with a protractor or a folded square of paper (a diagonal fold is 45°).

- For the 20° tilt: plane or sand the block's underside so it leans forward, or put a 5 mm shim under the back edge when you fix it down. Adjust later on the bench when the C7 calibration reports tilt (fix P5).

- Fix each block at its corner with hot glue or foam tape. Sit the board in the slot with a drop of hot glue at each end of the slot on the back side of the board only (never on the blue module side).

### Both options

- Sensor A goes back-left, sensor B back-right, as you face the sink to use it.

- The blue module faces the sink. The USB-C port ends up at the top. The header pins with wires come out of the back of the board.

- Check both boards are upright (not leaning sideways) and at the same height. Measure from the counter to the module centre on each side; they must match within 20 mm.

- Measure and write down, for each sensor, the distance from the ring's inside back edge and from the inside side edge to the centre of the blue module, and its height above the ring top. You type these into Ring Studio in calibration step C2 later.

## Step 26 · Shield and backboard

The radar also sees behind itself, and things moving behind the sink can look like a hand. Two measures stop that (review R16).

- Cut a piece of foil tape (or kitchen foil glued to card) about 30 × 45 mm for each sensor.

- Stick it behind each sensor board, covering the back of the board, on the mount or on a card spacer, at least 3 mm away from the header pins. The foil must never touch a pin or the solder joints. If the mount has a back wall, stick the foil to the outside of that wall.

- Check with the power off that no foil edge can reach a pin even if the board is pushed. Trim if needed.

- Stand the backboard (foam board or plywood, at least 60 × 40 cm) directly behind the rig's back edge, or push the rig's back edge against a wall. Nobody should be able to walk behind it during a demo.

## Step 27 · Route the cables

- Unplug the ESP32.

- Decide where the ESP32 will live: a dry spot behind or under the ring, within 50 cm of both sensors by cable, where its USB-C reaches a wall adapter.

- Replace the loose jumper wires with the Qwiic cables (or longer female-female jumpers): one 4-core cable per sensor for 3V3, G, SDA and SCL, plus a separate wire for RST. Keep each sensor's cable under 50 cm.

- Reconnect exactly as in the Step 17 table. Change the cables one sensor at a time, checking each of its five wires before starting the other sensor.

- Fix the cables down with tape or small clips so they cannot tug on the sensor pins when the rig is moved.

- Power up. Both PWR lights on. Load Ring Studio (once Phase 1 is on the board) and run C0 hardware check: it confirms both sensors answer and both reset lines work. If a sensor is not found: fix W1.

# Part 10 · Stage 2: the LED ring

You do this   ·   about 2 hours, once the parts arrive

The LED strip runs on its own 5 V supply. The ESP32 only sends it a data signal, through a small chip (the level shifter) that turns the ESP32's 3.3 V signal into the 5 V the strip expects. Keep this diagram beside you (docs/wiring-stage2-leds.png):

Stage 2: level shifter, strip, power supply and fuse. Every orange 5V tag joins the 5V rail; every black GND tag joins the GND rail.

Safety Everything unplugged and the LED supply switched off until Step 41. The LED supply's 5 V must never touch the ESP32's 5Vin or 3V3 pins, or any sensor pin. Only its ground joins the ESP32's ground.

## Step 36 · Fit the chip in the breadboard

Breadboard basics: the two long rows of holes along each edge are rails; every hole in a rail is connected along its length (some boards split the rail in the middle, check for a gap in the coloured line). The short rows of 5 holes in the middle are connected within each row of 5, and the centre gap separates the two halves.

- Choose the pair of rails on one edge. With a marker, label the outer rail 5V (it usually has a red line) and the inner rail GND (blue or black line). If the rails are split in the middle, bridge each half with a short jumper so the whole rail is one.

- Take the 74AHCT125 chip. Find the notch (a small half-moon cut-out) in one short end, and the small dot in one corner beside it. Pin 1 is the pin next to the dot.

- Hold the chip with the notch pointing up (away from you). Pin 1 is then top-left. Pins count down the left side 1 to 7, then up the right side 8 to 14. Pin 14 is top-right.

- Straddle the chip across the centre gap of the breadboard so seven pins are in rows on the left half and seven in rows on the right, notch up. Press it down evenly until it seats. Each chip pin now has four free holes in its row.

- Write down which breadboard row number each pin landed in (the rows are numbered along the edge). It saves counting later.

## Step 37 · Tie off the chip's pins

Every input on the chip must be connected to something, or it picks up noise. Use short male-male jumper wires from the free holes in each pin's row to the named rail.

| Chip pin | Name | Goes to | Tick |
| --- | --- | --- | --- |
| 1 | 1OE | GND rail | ☐ |
| 2 | 1A | the ESP32 data wire and the 10 kΩ pull-down (Step 39) | later |
| 3 | 1Y | the 330 Ω resistor to the strip (Step 39) | later |
| 4 | 2OE | 5V rail | ☐ |
| 5 | 2A | GND rail | ☐ |
| 6 | 2Y | nothing (leave empty) | ☐ |
| 7 | GND | GND rail | ☐ |
| 8 | 3Y | nothing (leave empty) | ☐ |
| 9 | 3A | GND rail | ☐ |
| 10 | 3OE | 5V rail | ☐ |
| 11 | 4Y | nothing (leave empty) | ☐ |
| 12 | 4A | GND rail | ☐ |
| 13 | 4OE | 5V rail | ☐ |
| 14 | VCC | 5V rail | ☐ |

- Do the GND ties first: pins 1, 5, 7, 9, 12 to the GND rail. Tick each.

- Then the 5V ties: pins 4, 10, 13, 14 to the 5V rail. Tick each.

- Count again against the diagram: five wires to GND, four to 5V, three pins empty (6, 8, 11), two pins waiting (2, 3).

## Step 38 · Prepare the strip

- Unroll the 5 V strip. Find the input end: the printed arrows point away from it. Its first pad set reads DIN (or DI), not DO.

- Measure the ring's outer perimeter where the strip will sit. Design value: about 2.2 m = 132 LEDs at 60 LEDs/m. Count LEDs, not centimetres, and cut at the cut point after the LED you want to be the last one. Cut exactly across the middle of the copper pads so both sides keep half a pad.

- At the input end, strip 5 mm of insulation from: one red 18 AWG wire, one black 18 AWG wire, and one thin green (or any colour) hookup wire about 15 cm long. Twist each wire's strands and tin them (touch the iron and a little solder to the strands so they are coated).

- Tin the pads: touch the iron to a pad for 1 second and add a little solder so a small shiny bump sits on it. Do the three pads. Do not hold the iron on a pad for more than 3 seconds at a time; the pads lift if overheated.

- Solder the wires to the input end, one at a time. Hold the tinned wire on the tinned pad, touch the iron to both for 1 to 2 seconds until the solder flows together, remove the iron, hold still for 3 seconds:

  - red 18 AWG to 5V;

  - black 18 AWG to GND;

  - thin green to DIN.

- Pull gently on each wire. None must move. Check no solder bridges two pads (look between them with a light; scrape with a knife tip if a thread of solder crosses).

- At the far end, solder a red 18 AWG to 5V and a black 18 AWG to GND the same way. Leave DO empty.

- The capacitor (1000 µF): it has a stripe with minus signs down one side and one leg is shorter; that is the negative leg. Trim both legs to about 10 mm.

- Solder the capacitor's striped (negative) leg to the GND pad and its other leg to the 5V pad at the input end, alongside the wires already there (or to the wires themselves 2 cm from the strip). A capacitor in backwards can burst, so check the stripe is on the GND side before you solder.

- Cover each soldered end with heat-shrink or two layers of electrical tape so the pads cannot touch anything metal on the ring.

## Step 39 · Make the data connections

- D1 (data in): a female-to-male jumper from the ESP32's left header pin 4 (GPIO4) to a free hole in chip pin 2's row. Count: left header from the top, 1 = 3V3, 2 = 3V3, 3 = RST, 4 = GPIO4. The board prints 4 beside it.

- Pull-down: bend the 10 kΩ resistor's legs (bands brown-black-orange) and push one leg into another free hole in chip pin 2's row and the other leg into the GND rail. It holds the data line quiet while the ESP32 starts, so the ring cannot flash random colours at power-up (fix L7).

- D2 (data out): push one leg of the 330 Ω resistor (bands orange-orange-brown) into a free hole in chip pin 3's row. Solder or firmly twist-and-tape the strip's thin green DIN wire to the resistor's other leg, and insulate the joint with heat-shrink. Keep the run from the chip to the strip under 15 cm; put the breadboard near the strip's input end.

- Common ground: a female-to-male jumper from the ESP32's right header pin 21 (GND) to the GND rail. Pin 21 is the second pin from the bottom on the right header; the board prints GND (or G) beside it. The ESP32 has four GND pins, all joined inside: W3 uses left pin 22 and W4 uses right pin 1, so this one is free. Without this wire the data signal has no reference and the LEDs stay dark (fix L1).

## Step 40 · Make the power connections

- Fuse: the inline fuse holder has two wire tails. Join one tail to the supply's positive (+) output wire: twist, solder and heat-shrink (or use a screw terminal block). Join the other tail to a red 18 AWG lead that goes to the 5V rail. Push the 5 A fuse into the holder.

- Supply negative (−) to the GND rail with a black 18 AWG lead.

- The breadboard rails accept only thin wire. For each 18 AWG lead, tin the end and push it firmly into a rail hole, or better, use a small screw terminal block as the rail junction: supply + and fuse on one side, the two red strip feeds and a thin jumper to the breadboard 5V rail on the other. Same for GND.

- Strip input end: its red 18 AWG to the 5V junction, its black 18 AWG to the GND junction.

- Strip far end: its red 18 AWG to the 5V junction, its black 18 AWG to the GND junction. Feeding both ends keeps the far end bright and the colours true (fix L4).

- Check the supply's output polarity before connecting: a barrel-jack adapter has + and − marked; bare wires use red or a white stripe for +. If unsure, measure with a multimeter or send me a photo.

- Plug the LED supply and the ESP32's USB-C wall adapter into one switched power strip, switch off. Do not plug the ESP32 into the PC for this test; the wall adapter powers it.

## Step 41 · Check, then power up

- Go through the stage 2 diagram tag by tag. Every orange 5V tag on the diagram must be joined to your 5V junction; every black GND tag to the GND junction. Count them.

- Confirm: chip notch up; capacitor stripe to GND; no wire from the LED supply to any ESP32 pin except the one GND wire to right pin 21; fuse fitted.

- Switch the power strip on.

- Watch the strip for the first 5 seconds. Expected: dark, or a soft idle chase once the firmware is running. Wrong: full white, random colours or flicker. If wrong, switch off and go to fix L2 (or L7 for a flash only at start-up).

- After 1 minute, feel the supply, the fuse holder, the wires and the strip. Warm is fine. Hot is not. Hot: switch off and send me a photo of the hot part.

- If the strip stays dark once Ring Studio's C0 LED test should be lighting it: fix L1, then L5 if only part lights, L3 if the colours are wrong.

- Once Ring Studio is on the board, run C0 and answer its LED questions (white test, red/green/blue test, count test). It sets the colour order and LED count for you.

CHECKPOINT 8

Stop here. Send me the following and wait for my reply before you go on:

- a short video of the ring lighting up (any pattern);

- a photo of the breadboard from above;

- the LED count C0 reported, or the number you set.

# Appendix A · Fixes you may need in these parts

Short versions of the codes referenced above. The full guide with every code is docs/09-troubleshooting.md.

## Two techniques

### Re-seat

Pull a jumper wire fully off its pin, look at the pin (straight, clean), push the wire back on firmly until it stops. Loose jumpers cause more faults than anything else on a bench build.

### The swap test (is it the sensor, or the wiring?)

- Unplug the ESP32's USB.

- At the ESP32 end only, swap A's signal wires with B's: W5 with W7, W6 with W8, W9 with W10. Power wires stay.

- Plug in and run the check again.

- If the fault moves to the other sensor name, it follows the ESP32 side: the ESP32 pins or the wires at that end.

- If the fault stays with the same physical board, that board or its end of the wires is the problem. Try a spare board.

- Put the wires back before continuing.

## W1 · Sensor not found

Ring Studio (or the Phase 0 monitor) says a sensor gives no answer.

- Look at that sensor's PWR light. Off: go to W6 first.

- Unplug the ESP32. Check its four bus wires against the Step 17 table, counting from the top: A uses W5 (left 12 → SDA) and W6 (left 15 → SCL); B uses W7 (left 10 → SDA) and W8 (left 11 → SCL).

- Re-seat all four of that sensor's wires at both ends.

- Press the sensor's RST button once. Do not touch BOOT.

- Plug in and test again.

- Still failing: swap that sensor's SDA and SCL wires at the sensor end only and test again. If it now works, leave it and tell me.

- Check left header pins 13 and 14 are empty.

- Still failing: swap test. If the fault stays with the board, reflash it (all five wires off first, Step 13), or use a spare.

## W4 · Reset line not working

- Check W9 runs from GPIO6 (left pin 6) to sensor A's left-side bottom pin, and W10 from GPIO7 (left pin 7) to sensor B's.

- If the message says the other sensor restarted: swap W9 and W10 at the ESP32 end.

- Look at the RST solder joint: a shiny cone. If not, reheat 3 seconds with a touch of fresh solder.

- Test again.

## W6 · A sensor's PWR light is off

- Unplug. Check its 3V3 wire (W1 for A, W2 for B) runs from an ESP32 3V3 pin (left pins 1 and 2) to the sensor's 3V3 (right pin 8).

- Check its ground wire (W3 for A, W4 for B) runs from an ESP32 GND to the sensor's G (right pin 7).

- Re-seat both. Plug in.

- Still off: unplug, move that sensor's 3V3 and G wires onto the other (working) board, plug in. If the good board's light now goes off, the wires or ESP32 pins are at fault. If it lights, the first board is faulty: use a spare.

## W7 · Heat or a burning smell

- Unplug the USB and switch off the LED supply now.

- Wait 5 minutes. Do not touch hot parts.

- Look for:

  - the 74AHCT125 in backwards (notch must point up);

  - the capacitor in backwards (stripe must be on GND);

  - foil touching header pins;

  - a 3V3 or 5V wire on a GND pin;

  - LED 5 V wired to a sensor or ESP32 pin;

  - a solder bridge between two pins.

- Photograph both sides of every board and send them to me before powering up again.

## W8 · ESP32 not in Device Manager

- Try the other USB-C port on the ESP32. Only the COM/UART port shows as CH343.

- Try another cable. Many USB-C cables are charge-only.

- Try another USB port on the PC.

- Yellow triangle: install the CH343 driver (Step 9).

- Appears then vanishes every few seconds: the USB power is being cut. Unplug, pull all sensor wires off the ESP32 and try again. If it now stays, a wiring short is the cause: check W7's list before reconnecting.

## F4 · Sensor reflash problems (CubeProgrammer)

Won't connect:

- Interface must be UART, not ST-LINK or USB.

- Put the sensor into loading mode again right before clicking Connect: hold BOOT, press and release RST, release BOOT.

- Port: the sensor's COM number. None listed: install the CH340 driver (Step 9).

- Baudrate 115200, parity Even (the default).

- Close anything else using the port (VS Code Monitor, another CubeProgrammer window).

- Try another cable or USB port.

- The sensor must not be wired to the ESP32 while reflashing.

Connects but programming fails:

- Start address must be 0x08000000.

- The file must be i2c_distance_detector.bin for the XM125.

- Click Full chip erase (the eraser icon in the Erasing & Programming page), confirm, then program again.

## F5 · VS Code Upload to the ESP32 fails

- Could not open port / port busy: close the Monitor (trash-can icon on the terminal) and any other serial program, then Upload again.

- Failed to connect / Wrong boot mode: hold the ESP32's BOOT, press and release RST, release BOOT, click Upload again. After the upload, press RST once.

- Still failing: unplug, pull all sensor wires off the ESP32, try again. A wire on left pin 13 or 14 can block upload mode. Put the wires back carefully afterwards.

- No serial port found: use the COM/UART port and a data cable (W8).

- Red build errors: run git pull, then click Clean and Upload again. Still failing: copy the red text and send it to me.

## L1 · LEDs stay dark

- Check the arrows: the green data wire must be on the end they point away from.

- Check the ground wire from ESP32 right pin 21 to the GND rail.

- Check chip pin 1 is on GND and pin 14 on 5V (Step 37).

- Check the fuse and that the supply is on (most have a small light).

- Check the strip's pad labels say 5V, DIN and GND.

- Test again.

## L2 · Random colours or flicker

Causes: no level shifter, a data wire over 15 cm, the 330 Ω resistor missing, a weak ground, the capacitor missing. Fix each in turn and test again.

## L3 · Wrong colours (red shows as green)

Answer C0's colour test; Ring Studio sets the strip's colour order for you.

## L4 · Far end dim or yellowish

Feed 5V and GND at both ends with 18 AWG (Step 40). Test at full brightness.

## L5 · Only part of the ring lights

- Settings → LED count: set the real number.

- Find the last LED that lights. The joint or LED just after it is the fault: reflow that joint, or cut out that LED and bridge the gap with three short wires (5V, data, GND).

## L6 · ESP32 restarts when the LEDs get bright

The ESP32 must be powered only by its own USB-C. Check the grounds. Lower Settings → LED brightness cap. Check the supply is at least 4 A.

## L7 · LEDs flash or show white while the ESP32 starts

Fit the 10 kΩ pull-down from chip pin 2 to the GND rail (Step 39).

## U1 · Can't see or join ArtesianRing (status light never lights)

If the ESP32's RGB status light never comes on, check the RGB solder pads next to it are bridged (Step 19). Wi-Fi details are in the main guide, Step 30.

## U7 · git pull refuses to update

Your local changes would be overwritten: a project file was changed on your PC. Keep my version and set yours aside:

cd "$HOME\code\artesian-ring-poc" git stash git pull

Anything else (for example untracked working tree files would be overwritten): copy the whole message and send it to me. Do not delete files to get past it.

# Appendix B · Pin reference

Boards held component side up, the way the diagrams draw them. Positions count from the top of each header. Bold rows are used in this build.

## ESP32-S3 (YD-ESP32-S3, DevKitC-1 layout), left header

| Position | Printed label | Used for |
| --- | --- | --- |
| 1 | 3V3 | W1 → sensor A 3V3 |
| 2 | 3V3 | W2 → sensor B 3V3 |
| 3 | RST |  |
| 4 | GPIO4 | D1 → level shifter pin 2 (stage 2) |
| 5 | GPIO5 |  |
| 6 | GPIO6 | W9 → sensor A RST |
| 7 | GPIO7 | W10 → sensor B RST |
| 8 | GPIO15 |  |
| 9 | GPIO16 |  |
| 10 | GPIO17 | W7 → sensor B SDA |
| 11 | GPIO18 | W8 → sensor B SCL |
| 12 | GPIO8 | W5 → sensor A SDA |
| 13 | GPIO3 | keep empty (strapping pin) |
| 14 | GPIO46 | keep empty (strapping pin) |
| 15 | GPIO9 | W6 → sensor A SCL |
| 16 | GPIO10 | reserved: sensor C SDA (future) |
| 17 | GPIO11 | reserved: sensor C SCL (future) |
| 18 | GPIO12 | reserved: sensor C RST (future) |
| 19 | GPIO13 |  |
| 20 | GPIO14 |  |
| 21 | 5Vin | never connect the LED supply here |
| 22 | GND | W3 → sensor A G |

## ESP32-S3, right header

| Position | Printed label | Used for |
| --- | --- | --- |
| 1 | GND | W4 → sensor B G |
| 2 | TX |  |
| 3 | RX |  |
| 4 | GPIO1 |  |
| 5 | GPIO2 |  |
| 6 | GPIO42 |  |
| 7 | GPIO41 |  |
| 8 | GPIO40 |  |
| 9 | GPIO39 |  |
| 10 | GPIO38 |  |
| 11 | GPIO37 |  |
| 12 | GPIO36 |  |
| 13 | GPIO35 |  |
| 14 | GPIO0 | onboard BOOT button (not wired) |
| 15 | GPIO45 |  |
| 16 | GPIO48 | onboard RGB status light (not wired) |
| 17 | GPIO47 |  |
| 18 | GPIO21 |  |
| 19 | GPIO20 |  |
| 20 | GPIO19 |  |
| 21 | GND | stage 2 → GND rail (LED supply ground) |
| 22 | GND |  |

## SparkFun XM125, right header (USB end at the top)

| Position | Label | Used for |
| --- | --- | --- |
| 1 | BOOT | leave unconnected |
| 2 | IO1 | leave unconnected |
| 3 | IO0 | leave unconnected |
| 4 | TX | leave unconnected |
| 5 | RX | leave unconnected |
| 6 | ADDR | leave unconnected |
| 7 | G | ground wire (W3 for A, W4 for B) |
| 8 | 3V3 | 3V3 wire (W1 for A, W2 for B) |
| 9 | SDA | SDA wire (W5 for A, W7 for B) |
| 10 | SCL | SCL wire (W6 for A, W8 for B) |
| 11 | WU | no wire |

## SparkFun XM125, left header

| Position | Label | Used for |
| --- | --- | --- |
| 1 | INT | leave unconnected |
| 2 | 3V3 | leave unconnected |
| 3 | 1V8 | leave unconnected |
| 4 | VU | leave unconnected |
| 5 | G | leave unconnected |
| 6 | G | leave unconnected |
| 7 | SIO | leave unconnected |
| 8 | CLK | leave unconnected |
| 9 | RST | solder one pin here; RST wire (W9 for A, W10 for B) |

## 74AHCT125 level shifter (DIP-14, notch up, pin 1 top-left)

| Pin | Name | Connection |
| --- | --- | --- |
| 1 | 1OE | GND rail |
| 2 | 1A | D1 from ESP32 GPIO4 and the 10 kΩ pull-down to GND |
| 3 | 1Y | 330 Ω → strip DIN |
| 4 | 2OE | 5V rail |
| 5 | 2A | GND rail |
| 6 | 2Y | empty |
| 7 | GND | GND rail |
| 8 | 3Y | empty |
| 9 | 3A | GND rail |
| 10 | 3OE | 5V rail |
| 11 | 4Y | empty |
| 12 | 4A | GND rail |
| 13 | 4OE | 5V rail |
| 14 | VCC | 5V rail |

# Appendix C · Checkpoints and what I do in parallel

| Checkpoint | After | You send | I reply with |
| --- | --- | --- | --- |
| 1 | Step 3 | strip pad photos, LEDs per metre, supply sticker and connector | which strip, whether the supply is enough, order changes |
| 2 | Step 11 | install confirmation, COM numbers, firmware file path | go-ahead for Part 3 |
| 3 | Step 14 | both reflashes complete (or a screenshot) | go-ahead for Step 15 and Part 4 |
| 4 | Step 19 | wiring photo from above, PWR lights, RGB pads | the Phase 0 firmware (Part 5) and its loading steps |
| 5 | Step 23 (main guide) | the bench test results T1 to T4 | go-ahead for Part 6; the mount design after your Step 24 measurements |
| 8 | Step 41 | ring video, breadboard photo, LED count | the C0 LED test results check |

## What I am building while you do these parts

- Ring Studio, the live site (Part 11): Showcase screen, Operator view, the full calibration studio C0 to C14 with guided fixes, demo loop, record and replay, sound, hand markers, all three layouts and the tests screen. It runs on a simulated hand first, so you can open it on your laptop before any hardware is wired.

- Phase 0 firmware (Part 5): the sensor driver and the echo printer for bench tests T1 to T4.

- Phase 1 firmware (Part 7): position maths, the latch state machine, calibration storage, Wi-Fi and the web server that serves Ring Studio from the ESP32.

Each push comes with a message saying exactly which buttons to click. Loading is always the same: git pull, then in VS Code esp32s3 → General → Upload, then Platform → Upload Filesystem Image (main guide Steps 21 and 29).